# Feeding Nimo — Setup Guide

Welcome! This guide will walk you through setting up your Feeding Nimo aquarium monitor. The whole process takes about 10 minutes.

# What you have

- The Feeding Nimo device — an automatic fish feeder with built-in water sensors (pH and TDS) and a food-level detector.

- The Feeding Nimo web app — your dashboard for checking water quality, viewing trends, and controlling the feeder.

# What you’ll need

- A WiFi network the device can stay connected to (2.4 GHz; most home routers work)

- Your WiFi password

- A phone, tablet, or laptop with a web browser

# Step 1 — Open the web app

On any device, open this link in your browser:

https://feeding-nimo.vercel.app

Bookmark it — this is your dashboard from now on.

# Step 2 — Power on the Feeding Nimo

Plug the device in. After a few seconds, it will create its own temporary WiFi network so you can give it your home WiFi details.

You’ll know it’s ready when you can see a new WiFi network called Feeding Nimo Setup on your phone.

# Step 3 — Connect your phone to the device’s setup network

On your phone or laptop, open WiFi settings and join:

- Network name: Feeding Nimo Setup

- Password: feedingnimo

Your phone may say “no internet” while connected. That’s expected — you’re talking directly to the device.

# Step 4 — Provision the device

- With your phone still connected to Feeding Nimo Setup, open the web app and tap Settings (bottom right).

- Under Device Setup, leave the URL as http://192.168.4.1.

- Tap Scan. The device will list nearby WiFi networks.

- Select your home WiFi from the dropdown.

- Enter your WiFi password.

- Leave API Base as https://feeding-nimo.vercel.app.

- Leave Device ID as esp32-001 (unless you’ve been given a different one).

- Tap Complete Device Setup.

The device will save your settings, reboot, and connect to your home WiFi automatically.

# Step 5 — Reconnect to your home WiFi

Switch your phone back to your normal WiFi network and refresh the web app.

Within a couple of minutes, the dashboard should show ONLINE with a fresh reading.

# Using the app

## Monitor (main page)

An at-a-glance view of your aquarium:

- pH — water acidity. Safe range 6.5–8.5.

- TDS — total dissolved solids in ppm. Lower is cleaner; 0–500 ppm is good for most freshwater tanks.

- Food level — how much fish food is left in the dispenser.

Colored rings show whether each reading is in the safe (teal), warning (orange), or danger (red) range.

## Graphs

Tap Graphs to see how pH, TDS, and food level have changed over time. Useful for spotting slow trends.

## Feeder

- Feed Now — dispenses one standard portion immediately.

- Feeding Schedules — set daily times for automatic feeding. Add as many as you want; toggle them on/off anytime.

## Settings

- Change device ID, refresh interval, and dark mode.

- Re-run device setup if you ever move the feeder to a new WiFi network.

# Troubleshooting

### Dashboard says OFFLINE / readings are stale

- Check the device is powered on.

- Check your home WiFi is working and the device is within range.

- If you changed your WiFi password recently, re-run setup (Step 4).

### Can’t find the Feeding Nimo Setup WiFi network

- Power-cycle the device: unplug, wait 10 seconds, plug back in.

- The setup network only appears when the device has no saved WiFi. If it’s already connected, you won’t see it.

- To force setup mode, hold the SET button while powering on, or contact your developer for a factory reset.

### Feed Now button doesn’t dispense food

- The device polls for feed commands every ~10 seconds, so there’s a short delay.

- Make sure the device shows ONLINE on the dashboard.

- Check the food container isn’t empty.

### Readings look wrong

- Sensors need to be submerged and given a few minutes to stabilize after first use.

- pH sensors drift over time and may need recalibration every few months.

# Need help?

Contact your developer if anything doesn’t work as expected. When reporting an issue, please include:

- What you were doing

- What the dashboard showed

- Whether the device’s status light was on