# Feeding Nimo — Developer Guide

Internal reference for the AquaWatch / Feeding Nimo project. Stack, deploy flow, and gotchas.

# Live URLs

| Resource | URL / Identifier |
| --- | --- |
| Web app + API | https://feeding-nimo.vercel.app |
| Vercel project | eewonns-projects/aqua-watch-backend |
| Supabase project | mxyqlgdzerauefogeqrf |

# Repository layout

aqua-watch/

├── api/                  # Next.js 16 web app (frontend + backend)

│   ├── app/              # Pages + /api/* route handlers

│   ├── lib/              # Supabase client, ranges, client-api

│   └── vercel.json       # Pins framework=nextjs

├── mobile/               # Expo app (NOT deployed; legacy parallel UI)

├── feeding-nimo code/    # PlatformIO ESP32 firmware

│   └── src/main.cpp      # WiFi provisioning, sensors, API integration

├── supabase/schema.sql   # 4 tables

└── docs/

The folder named “api/” is actually the entire Next.js app (frontend + API). Don’t be misled by the name.

# Stack

- Next.js 16 — App Router, React 19, React Compiler enabled

- Supabase (Postgres) — service-role key on server; no auth on requests (device_id only)

- Tailwind CSS v4

- Firmware: PlatformIO + Arduino framework on ESP32

- Firmware libs: HTTPClient, WebServer, ArduinoJson, ESP32Servo, NTPClient, LiquidCrystal_I2C, Adafruit_SSD1306, HX711, Preferences

# Common commands

## Web app

cd "Personal Projects/aqua-watch/api"

npm run dev                     # local dev

npx tsc --noEmit                # type check

npx vercel --prod --yes         # deploy to production

npx vercel env pull .env.local  # sync env vars locally

## Firmware

cd "Personal Projects/aqua-watch/feeding-nimo code"

pio run                         # build

pio run -t upload               # flash over USB

pio device monitor              # serial logs

# Environment variables (Vercel)

| Name | Scope | Notes |
| --- | --- | --- |
| NEXT_PUBLIC_SUPABASE_URL | Prod / Preview / Dev | Public; safe in browser |
| SUPABASE_SERVICE_ROLE_KEY | Prod / Preview / Dev | Server-only; full DB access |

Updating env vars does not auto-redeploy. Run npx vercel --prod --yes after every change.

# API contract

Used by both the firmware and the web UI. Same origin for both.

| Method | Path | Purpose |
| --- | --- | --- |
| POST | /api/readings | ESP32 posts {device_id, ph, tds, food_level} every 5 min |
| GET | /api/readings?device_id= | Latest reading (Monitor page) |
| GET | /api/readings/history?device_id=&limit= | Graphs page |
| POST | /api/feed | Manual FEED NOW |
| GET | /api/commands/pending?device_id= | ESP32 polls every ~10s |
| PATCH | /api/commands/[id]/execute | ESP32 marks done |
| GET/POST | /api/schedule?device_id= | Read/add feed schedules |
| PUT/DELETE | /api/schedule/[id] | Edit/remove schedule |

# Firmware provisioning contract

When the ESP32 has no saved WiFi, it boots into AP mode and serves a tiny config server.

- SSID: Feeding Nimo Setup

- Password: feedingnimo

- Captive IP: 192.168.4.1

| Method | Path | Body / Response |
| --- | --- | --- |
| GET | /networks | Returns [{ssid, rssi, secure}, ...] |
| POST | /configure | Body: {ssid, password, api_base, device_id}. Saves to NVS, reboots in 1s. |

The Settings page (api/app/settings/page.tsx) calls these endpoints from the user’s browser while joined to the Feeding Nimo Setup hotspot. CORS is allowed by the firmware.

# ESP32 pin map

| Component | Pins |
| --- | --- |
| pH sensor | A0 analog, Serial2 RX=16 TX=17 |
| TDS sensor | 34 |
| Servo | 27 |
| Buttons (UP / SET / DOWN) | 12 / 13 / 14 |
| HX711 load cell | DOUT=18, SCK=19 |
| Food-level LEDs (LOW / MED / HIGH) | 25 / 26 / 33 |
| I2C (OLEDs + LCD) | SDA=21, SCL=22 |
| OLED pH / TDS addresses | 0x3C / 0x3D |
| LCD address | 0x27 |

# Gotchas

- Folder name api/ is misleading — it’s the full Next.js app, not just the API.

- Vercel project Framework Preset is set to “Other” in the dashboard. api/vercel.json overrides this to “nextjs” for CLI deploys. Don’t delete that file.

- Supabase free tier auto-pauses after ~1 week of no traffic. If the API returns fetch failed, check Supabase dashboard and restore the project.

- Deployment protection is currently off. If it gets re-enabled, the public alias may serve 401s.

- No auth on the API — anyone with a device_id can read/write. Acceptable for a research project, not for production.

- Single deployment ships frontend and /api/* together; you can’t redeploy them independently.

# Deploying a fix

- cd "Personal Projects/aqua-watch/api"

- Edit files.

- Run npx tsc --noEmit as a sanity check.

- Run npx vercel --prod --yes (takes ~30s).

- Verify with: curl https://feeding-nimo.vercel.app/api/readings?device_id=esp32-001

Rollback: Vercel dashboard → Deployments → pick a previous Ready build → Promote to Production.

# What’s still TODO

- End-to-end test with real ESP32 hardware on the client’s WiFi

- Optional: build the mobile Expo app as an APK via EAS (the web app already covers the same UI)

- Add basic auth or a shared secret if scope expands beyond one device

- Calibrate sensors before handing the device to the client